UniCGA Version 1.0: Universal CGA simulator             (c) 1991 Tommy Frandsen

-------------------------------------------------------------------------------

CGA simulator for the Hercules Graphics Card

Features:

- Supports all CGA graphics and text modes

- Automaticly detects the correct CGA mode

- Four different simulation methods

- Adjustable refresh rate

- Has been succesfully tested with many applications and games

Requirements:

- An IBM PC/XT/AT/Clone

- MS-DOS/PC-DOS 2.0 or higher

- A Hercules Graphics Card/Clone

Syntax: UCGA [ -c | -cxxx | -f | -s | -sxxx | -d | -u | -h ]

Description:

UCGA -c    : hardware simulation, 240 refreshes/min.

UCGA -cxxx : hardware simulation, xxx refreshes/min.

UCGA -f    : hardware simulation, no refresh (fast)

UCGA -s    : software simulation, 180 refreshes/min.

UCGA -sxxx : software simulation, xxx refreshes/min.

UCGA -d    : software simulation, DOS controlled refresh

UCGA -u    : disable simulation and return to text mode

UCGA -h    : show help-screen

Comments:

- The "-f" option (hardware simulation without refresh) is very fast but

produces annoying blank lines. The "-c" option (hardware simulation

with timed refresh) is slower but eliminates the blank lines

- The "-s" option (software simulation with timed refresh) is the slowest

simulation method but it produces the most pleasant display

- The "-d" option (DOS controlled refresh) will only work with programs

that uses DOS for input and output (like COMMAND.COM)

- Use the DOS MODE command to switch between 40 and 80 column text mode

- Use the DOS GRAFTABL command to load graphics characters (ASCII 128-255)

Warnings:

- Do not run UCGA if there is no Hercules Graphics Card installed

- Do not run UCGA if there is more than one graphics card installed

- Although this program has been very troughfully tested, any usage of

UCGA is totally at your own risk. I am not responsible for any damage

caused by the program, either direct or indirect, to your hardware

or software. If you can't accept this restriction, please don't use

UCGA.

-------------------------------------------------------------------------------

You may freely copy and distribute this program. If you find UniCGA useful

and convenient, a donation of $5 would be appreciated. Please send check to:

Tommy Frandsen

Lovtoften 50

DK-2630 Taastrup

Denmark

-------------------------------------------------------------------------------